8 principles of programming on FPGA

- MAXIMUM COMMON CLOCKS. All clocked elements such as registers, counters, FSM, individual triggers and RAM should be clocked with a common sync signal.

- CLK TO CLK. Apply a sync signal only to the sync inputs of triggers and nowhere else.

- COMBINATION LOOPS. Feedbacks in combinational circuits are prohibited.

- ASYNCHRONOUS SIGNALS. Do not use asynchronous signals. All asynchronous signals entering the circuit must be synchronized using at least 2 flip-flops.

- CLOCKS THE SAME TIME. The clock signal for all clocked elements must arrive at the same time.

- COMB TO TRIG. Connect the combinational circuit only to the D input of the trigger and nowhere else.

- REG OUTPUT. The output of the combinational circuit arriving at the external device is passed through the trigger.

- ASYNCHRONOUS RESETS. Use asynchronous flip-flop resets only for general circuit reset.